A Transferable Computational Framework for Modeling and Tuning Biomimetic Catalysis

Thomas Quaid1, Cooper Citek2, John F. Hartwig2,3, Douglas S. Clark1,4*

1Department of Chemical and Biomolecular Engineering, University of California, Berkeley, California, 94720, United States

2Chemical Sciences Division, Lawrence Berkeley National Laboratory, Berkeley, California, 94720, United States

3College of Chemistry, University of California, Berkeley, California, 94720, United States

4Molecular Biophysics and Integrated Bioimaging Division, Lawrence Berkeley National Laboratory, Berkeley, California, 94720, United States

# *To whom correspondence should be addressed (dsc@berkeley.edu)

# GRAPHICAL ABSTRACT

# ABSTRACT

Carbonic anhydrase (CA) catalyzes CO₂ hydration with turnover frequencies approaching 10⁶ s⁻¹, but native enzymes lack stability under conditions typical of industrial carbon capture. Small-molecule zinc mimics offer an alternative, yet product inhibition by bicarbonate limits their efficiency and no general framework exists to predict turnover across diverse ligand scaffolds. We report the analysis, by a combination of experiment, computation, and data science, of the speciation and catalytic activity of a series of zinc complexes as mimics for carbonic anhydrase. We created a data set encompassing eleven Zn–ligand complexes spanning macrocyclic, tripodal, and monodentate classes and calculated the binding free energy (ΔGbind) of bicarbonate to zinc with DLPNO-CCSD(T) refinement. ΔGbind alone predicts intrinsic rate constants for CO₂ hydration with R² = 0.90 at pH 9, 25 °C, and saturated CO₂. First-sphere descriptors define a quantitative activity ceiling of ~10³ M⁻¹-s⁻¹ for classical ---multidentate scaffolds. Monodentate imidazole assemblies are not bound by this constraint; the predicted intrinsic -rate for ZnL₄(OH) (~1490 M⁻¹s⁻¹) exceeds the highest reported experimental -value. Observed rates for monodentate systems are an order of magnitude lower, a discrepancy explained by equilibrium speciation: even at the optimal 15:1 ligand-to-zinc ratio, only ~32% of zinc exists in catalytically active forms. The resulting correction factor Φspec ≈ 0.05 enables rate prediction without system-specific equilibrium constants. Within the N-functionalized cyclen series, ground-state steric congestion (%Vburied) overrides ΔGbind in determining activity rank, attributed to the inability of macrocyclic scaffolds to reorganize rapidly during catalysis. Together, these results comprise a transferable computational framework integrating quantum descriptors and chemical speciation for predictive design of zinc-based CO₂ hydration catalysts.

Keywords

carbonic-anhydrase mimic; zinc-mediated CO₂ hydration; speciation-corrected modeling; ΔGbind descriptor; outer-sphere stabilization; macrocyclic ligands; imidazole excess

# INTRODUCTION

Metalloenzymes are providing inspiration for the development of synthetic molecular inorganic catalysts that react with high activity and selectivity under mild conditions.1–3 A standard bearer of metalloenzymatic efficiency is carbonic anhydrase (CA), which catalyzes the hydration of carbon dioxide with turnover frequencies approaching 10⁶ s⁻¹.4,5 These turnover frequencies are unmatched by synthetic analogs. The active site of CA is a tetrahedral Zn²⁺ center coordinated by three histidine residues and a water molecule, which upon deprotonation, forms a highly reactive Zn–OH species.6,7 Despite their favorable catalytic properties, the stability of native CAs is poor under alkaline and thermally stressful conditions that are typical of industrial carbon capture processes.8 Consequently, robust small-molecule mimics have been sought that could operate in these environments without loss of structural or functional integrity.9

The pursuit of abiotic CA mimics spans more than half a century, from early efforts involving cobalt complexes of tetra-azacyclododecane or phosphines10,11 to more elaborate zinc complexes engineered to engender second-sphere hydrogen-bonding interactions.12,13 However, product inhibition, due to the accumulation of bicarbonate, remains a prevailing challenge.14 Rains et al.14 introduced ZnL1S (structure available in Figure S4) as a high-performance mimic, reporting reaction rate constants that change sharply with varying concentrations of CO₂ and, therefore, changing concentrations of the bicarbonate inhibitor. Unfortunately, these conditions deviate from practical levels of CO2 and mask the intrinsic limitations of the catalyst. It is now well understood that the rate-limiting step in most small-molecule systems is the release of bicarbonate,14,15 not CO₂ activation, thereby necessitating new strategies beyond tuning of the primary Zn-coordination sphere.

Ab initio property predictions of thermodynamic and kinetic properties of molecular systems have improved substantially in accuracy and computational efficiency over the past decade. The rapid exploration of chemical spaces made possible by computations that augment experiments has fueled many advances in catalysis.16–18 As a result, significant progress has been made in the rational design of biomimetic catalysts across diverse applications.19,20 Descriptor-based computational modeling has provided a framework to interrogate and predict the performance of CA mimics by reducing computational costs of evaluating complex systems.21,22 Density-functional theory and local-correlation methods have enabled the quantification of thermodynamic and electronic parameters, such as the Zn-bicarbonate binding energy (ΔGbind), metal-centered charge (qZn),23 donor–acceptor interaction energies (ΣE(2)),24 and the steric accessibility of the active site (%Vburied).23,25–28 Through strategic combinations of these computational approaches, accurate modeling can now be applied to systems that are generally recalcitrant to rigorous applied methods such as WFT (wave function theory) to allow for significant increases in predictive accuracy, as demonstrated in this work.

In the present work, we introduce a protocol for modeling and evaluating the catalytic efficiency across a structurally diverse array of zinc coordination motifs that is based on molecular descriptors and corrected for speciation of the zinc complexes. A high-throughput automation workflow we term QUAntum woRKflow Suite (QUARKS) automates the creation of bulk 3D structure files and automatically manages large computation workloads for conformer searches, geometry optimizations, energy refinements, and descriptor extraction used in this work for large sets of ligands across multiple ligand classes. The suite of programs includes several open access programs like ORCA 29 and Molassembler 30 with newly generated graphical user interfaces (GUIs).

Conventional implementations of modeling for CA mimics have often failed to account for speciation, particularly for systems in which complexes are formed from monodentate ligands. Our findings demonstrate that ligand stoichiometric excess can be modeled quantitatively to predict observed rates, providing a design variable beyond ligand identity alone. By integrating quantum mechanical modeling, speciation corrections (log K’s and Henderson-Hasselbalch computations), and experimental rate constants, this study presents a reproducible, transferable framework for predicting the catalytic performance of zinc complexes in CO₂ hydration and, potentially, other systems where product dissociation is rate-limiting. Furthermore, we create a systematic, benchmarked, and experimentally validated computational approach to studying multidentate ligand systems and use this system to predict the activity of metalloenzyme mimics. These findings establish a validated computational framework for designing small-molecule CA mimics, where catalytic activity is limited by bicarbonate dissociation and modulated by active catalyst speciation of the zinc complex.

# COMPUTATIONAL STRATEGY

All quantum chemical and thermodynamic calculations were performed using a descriptor-based high-throughput platform developed internally and implemented as the QUARKS pipeline. Ligands were first constructed in Avogadro31 and then subjected to a conformational search using GFN2-xTB32 and CREST.33 Geometry optimizations were performed at the B3LYP-D4/def2-SVP level using CPCM solvation (water), followed by frequency calculations and single-point energy calculations at B3LYP-D4/def2-TZVPP. A visual protocol is available in Figure S1. All illustrated chemical structures in the manuscript and the supplementary information were produced with either ChemDraw34 or Avogadro.

To develop an accurate procedure for modeling zinc complexes, we curated a benchmark dataset comprising the activity of zinc complexes of five-ligands (M1–M5) reported by Koziol et al.,15 selected for internal consistency and the reporting of experimental rate constants. This training set allowed us to isolate method performance without inter-laboratory variability. Ten DFT procedures were tested, spanning functionals, basis sets, dispersion corrections, and solvation treatments desccribed in Table S1. For each procedure, the Zn–bicarbonate binding energy (ΔGbind) was computed and correlated with the experimental rate constant (kobs) for M1-M5.

Each literature kobs value in this study was converted to the intrinsic rate constant (kint) through the procedures outlined in the supplementary information section titled “pH correction of rate constants” page 7.

Among the protocols tested, B3LYP/def2-TZVPP, D4 dispersion correction, and CPCM(water) solvation led to a strong correlation between the computed ΔGbind values and  the experimental kint, with R² = 0.86, MAE = 532, RMSE = 617 M-1s-1, Q2LOO = 0.58 and MAECV = 940 M-1s-1, while retaining sufficient basis-set completeness and polarization flexibility to remain reliable across chemically diverse ligands. To further refine the energy accuracy, DLPNO-CCSD(T)/def2-TZVPP single-point corrections were applied to the DFT-optimized geometries and frequencies. This hybrid method achieved a strong correlation of R² = 0.91 between the experimental kint values and the computed carbonate binding energies for the benchmark dataset (Figure 1) and was used for all subsequent ligand screening and descriptor extractions.

15

Figure 1. Correlation between computed Zn–bicarbonate binding free energies (ΔGbind) and experimentally determined intrinsic rate constants (kint) for the five macrocyclic CA mimics M1–M5 reported by Koziol et al.15 The ΔGbind values were computed at the B3LYP‑D4/def2‑TZVPP level with DLPNO‑CCSD(T) single‑point refinement; kint values were reported for uniform assay conditions (pH 9, 25 °C, saturated CO₂). The dashed line shows the least‑squares fit (R² = 0.91), illustrating the inverse relationship between turnover and bicarbonate affinity that forms the basis for rate prediction in this work. Speciation effects were addressed by predicting ligand pKₐ values from the optimized Zn-OH bond lengths using a linear model trained on a combination of experimental and calculated values from Koziol et al. available in Table S2. The resulting fit, pKₐ = 43.34·(dZn–O) -74.42, reproduced benchmark pKₐ values with R² = 0.82 (Figure S1). The fraction of active catalyst (fOH) was then determined using the Henderson-Hasselbalch equation.

The final procedure and speciation model were validated using an independent dataset of eleven-ligands drawn from multiple literature sources (Table S5), excluding the five datapoints used in the benchmarking study (Figure 1). This external set encompasses a range of ligand classes and rate constants (converted to intrinsic values using the approach described in the caption of Figure 1 and Supplemental Information) spanning over an order of magnitude. The strong agreement between the predicted and experimental values across both the benchmark and validation sets forms the foundation for the modeling, screening, and mechanistic analyses presented in the following sections.

Steric parameters were computed to test whether ground-state congestion contributes to rate prediction beyond ΔGbind, as discussed in the Results section. Steric congestion was quantified by the SambVca protocol originally reported by Poater et al.35; a 4.0 Å sphere centered on zinc was selected after sensitivity analysis (2.5–5.0 Å) identified this radius as most accurately reproducing the experimental ranking of activities. Conformer ensembles generated with GFN2-xTB/CREST were Boltzmann-weighted at 298 K; the reported %Vburied values are ensemble averages. All scripts are available at github.com/QuaidtUC/Sterics_Study.

## THE CASE FOR STANDARDIZATION

The reported reaction rate constants for the hydration of CO2 catalyzed by identical Zn-cyclen and Zn-cyclam complexes span approximately one order of magnitude (Table 1), even among studies that used the same instrumentation, reagents, and stopped-flow indicator method. This divergence suggests that variations in data processing, rather than experimental setup, likely account for the discrepancy. The uncatalyzed rate constants (~ 0.12 s-1 measured in 36) provides an intrinsic benchmark that can be used as a consistency check to bridge conditions and methods. Frequently cited values for catalytic rate constants of ~3 × 10³ M⁻¹ s⁻¹ (and higher) are not directly comparable under standardized conditions used most often in published studies (pH ≈ 9, saturated CO₂, 25 °C, 1 mM zinc as upper limit). Considering this discrepancy, we emphasize the need for uniform data-processing and reporting criteria before meaningful performance rankings or mechanistic inferences can be derived. The data in Table 1 are presented in this context to establish a consistent basis for comparison.

Table 1. Selected rate constants (kobs) reported for CO2 hydration catalyzed by zinc-cyclen and zinc-cyclam complexes.

| Catalyst | Zhang et al [25] 1993 | Zhang et al [26] 1995 | Satcher Jr. et al [27] 2011 | Koziol et al [12] 2012 | This work 2025 |
| --- | --- | --- | --- | --- | --- |
| Zn‑Cyclen (M3) | 581 ± 64 | 3300 ± 100 | 471 ± 33 | 2493 ± 49 | 343 ± 47 |
| Zn‑Cyclam (M1) | – | – | 126 ± 4 | 690 ± 130 | 68 ± 8.8 |
| None | 0.12 ± 0.01 | – | – | – | 0.15 ± 0.02 |

# RESULTS AND DISCUSSION

## BENCHMARK VALIDATION

Using the computational protocol described above (see Computational Strategy), the data from Koziol et al. with ligands M1-M5 provided a rigorously controlled dataset (identical buffer, CO₂ loading, temperature, and zinc concentration) to calibrate our benchmarking study. Across ten candidate combinations of functional, basis set, solvation method, and dispersion corrections, the value of ΔGbind computed with B3LYP-D4/def2-TZVPP + DLPNO-CCSD(T) aligned with the intrinsic rate constants with a correlation of R² = 0.91. The computed ΔGbind values spanned 12 kcal/mol across the five ligands. This range, combined with the strong correlation to kint, is consistent with bicarbonate dissociation being the turnover-limiting step, as previously proposed by Koziol et al.

The transferability of the model to structurally diverse ligands proved equally strong: when the model trained on the five complexes of macrocycles was applied to set of 11 ligands (Figure 2, Table S5), the correlation between computed bicarbonate binding energy and measured rate constant remained high (R² = 0.90). This set includes varied ligand scaffolds such as macrocyclic tetraamine and tripodal scorpionates; Table S5 includes sources and values for kint, literature experimental kobs, pKa adjusted conversion to kint (normalized to pH 9) and computed binding energies from DFT and WFT. Table S7 includes all structures. These results confirm that ΔGbind is a transferable, physics-grounded predictor of the rates of zinc-catalyzed CO₂ hydration. Correlations of kint with binding energies for carbonate computed by DFT alone (B3LYP-D4/def2-TZVPP, CPCM(Water), D4) were poorer for this 11-ligand set. The statistical metrics for this correlation with binding energies computed with DFT + WFT (DLPNO-CCSD(T) single point energy) were better R2 (+12.5%), RMSE (-29%), and MAE (-34%). Metric comparisons are available in Table S6 of the Supplemental Information. This is a larger performance gap than that obtained in the benchmarking study. For example, for ligand Zn6c, ΔGbind shifted from 232.7 (DFT) to 10.04 kcal/mol (DFT+WFT), reflecting the substantial corrections from DLPNO-CCSD(T) single-point energies.

Figure 2. Correlation between predicted Zn–bicarbonate binding energy (ΔGbind) and experimentally derived intrinsic rate constants (kint) for CO2 hydration for a set of eleven literature‑reported zinc–ligand complexes. All data were recalibrated to standard conditions (pH 9, 25 °C, saturated aqueous CO₂, [Zn] = 1 mM) using the speciation normalization protocol. The strong inverse relationship (R² = 0.90) validates ΔGbind as a mechanistically grounded predictor of catalytic performance across diverse coordination environments. Error bars for the y‑axis are omitted due to inconsistent reporting and error handling from the original sources, which foster misleading inferences in comparisons. An image of the ligand with the highest literature kint (832 M-1s-1) to date is shown. Data, ligands, sources, and comparisons of DFT vs DFT+WFT derived binding energies are available in Table S5 of Supplemental Information.

Taken together, these results confirm that ΔGbind is a reliable descriptor for the catalytic activity of zinc-based CO₂ hydration mimics. The statistical differences between correlations with DFT and DFT+WFT outlined above make the latter method a strong candidate for generating training data for machine learning applications in CA mimic design, where the additional computational cost of WFT single-point corrections may be justified by significant improvements in accuracy. This benchmark performance provides a quantitative foundation for broader catalyst design and mechanistic investigations described in the following sections.

## INNER-SPHERE DESCRIPTORS REVEAL A QUANTITATIVE ACTIVITY CEILING

To probe descriptor based modeling beyond the general ΔGbind correlation (Figure 2, R² = 0.90), we evaluated three frequently used orthogonal inner-sphere descriptors for the eleven-ligand validation set (n = 11): (i) the NBO donor–acceptor second-order energy ΣE(2); (ii) the change in Zn NBO charge on binding, ΔqZn = qZn,complex – qZn,bare; and (iii) the change in buried volume, Δ%Vburied = %Vburied(bicarbonate) – %Vburied(aquo). For (i) and (ii) see work from Glendening et al.24 for derivations. The least squares regression obtained is given by Equation (1):

ln k = –11.65 + 0.15ΣE(2) – 43.9ΔqZn + 1.06%Vburied	(1)

The correlation between the normalized, pKa adjusted experimental ln k and predicted ln k using this equation was R² = 0.73, with a with a leave-one-out cross-validated R²CV = 0.42. The negative coefficient for ΔqZn (−43.9) indicates that accumulation of positive charge on Zn inhibits turnover, while positive coefficients for ΣE(2) (+0.15) and Δ%Vburied (+1.06) indicate that stronger donor interactions and increased steric bulk modestly enhance rates. To interpret the relative influence of each descriptor, we refit the regression using z-normalized inputs. ΣE(2), was the dominant factor (-0.36) determining the rate, followed by ΔqZn (-0.33) and Δ%Vburied (+0.26). The modest contribution of Δ%Vburied suggests that steric crowding around the zinc center favors product displacement. These inferences align with reported steric and electronic trends in small molecule ligand tuning for nickel based carbon dioxide fixation and zinc based ring-opening polymerization of lactides.37,38

This model defines quantitative limits on the rate constants achievable through first-sphere modifications of classical multidentate ligands. The values of the descriptors for the full set of complexes spanned the following ranges: ΣE(2) = 164–198 kcal mol⁻¹, ΔqZn = 1.66–1.77e, and Δ%Vburied = –2.8% to +6.4% (Table S3). The favorable extremes of the descriptors (ΣE(2) = 198 kcal mol⁻¹, ΔqZn = 1.66 e, Δ%Vburied = +6 %), lead to a value for ln(k) ≈ 6.8 (≈ 1 × 10³ M⁻¹ s⁻¹). This value is the maximum rate constant for the inner-sphere interactions involving these ligands. This ceiling indicates that these inner sphere effects alone are not sufficient to enable the complexes to approach enzymatic rates (10⁶ s⁻¹).

EXPERIMENTAL VALIDATION AND OVERPERFORMANCE OF MONODENTATE LIGANDS

Considering this ceiling in kint for reactions catalyzed by zinc complexes of multidentate ligands, we tested whether the ΔGbind descriptor also correlates with the values of kint for reactions catalyzed by complexes containing more labile monodentate ligands. A virtual screen of 29 monodentate N-heterocyclic ligand structures from the CHEMBL database39 was performed. The computed ΔGbind values were converted to predicted intrinsic rate constants (kint, pred) by means the linear correlation between ΔGbind and kint (Figure 2) derived from the training set comprising the eleven macrocyclic ligands. The resulting distribution (ranked by the predicted rate constant) is shown in Figure 3. Compound identifiers are provided in Table S4.

Figure 3. Intrinsic rate constants (kint, pred) predicted for 29 monodentate N-heterocyclic ligands identified from the CHEMBL database. The structure of each of the zinc carbonate complexes was modeled using the validated B3LYP‑D4/def2‑TZVPP + DLPNO‑CCSD(T) procedure, and the resulting ΔGbind values for carbonate binding were converted to predicted rates using the linear correlation established in Figure 2. Candidates were ranked in descending order of activity. Three ligands (compounds 4, 15, and 24 highlighted in grey and displayed in reported order as images) were selected for experimental validation. Corresponding CHEMBL identifiers and predicted rate values are listed in Table S4.

To evaluate the ability of this model to predict the reactivity of complexes containing monodentate ligands, three ligands containing imidazole units, 2-methylimidazole (4), imidazole (15), and 2,6-dichloropurine (24), were selected for experimental rate determination. These ligands were chosen because they represent the full range of predicted kint for the modeled ligands. Their relative observed rates, adjusted to intrinsic rate constants, precisely matched the order predicted by the model (Table 2). These results confirm the correlation of the catalytic rates to the carbonate binding energy for complexes of these monodentate ligands.

Table 2. Measured rate constants (kobs M⁻¹ s⁻¹), calculated experimental intercepts, and predicted intrinsic rate constants with speciation correction (kint,pred,speciation corrected M⁻¹ s⁻¹) for three CHEMBL‑sourced monodentate ligands and 1‑methyl imidazole for literature comparison (evaluated at a 4:1 ligand‑to‑zinc ratio and pH 9). Predicted intrinsic rate constant values (kint,pred M⁻¹ s⁻¹) were obtained by computing zinc–bicarbonate binding energies using the validated computational protocol and converting them via the linear fit established in Figure 2. The ranking of experimental rates matches the predicted order, yielding R² = 0.83 across the set. Intercepts reflect the extrapolated uncatalyzed rate, consistent with literature baselines for pH 9 under comparable CO₂ saturation.

| Ligand | kobs (M⁻¹ s⁻¹) | Intercept (s⁻¹) | kint,pred,speciation corrected (M⁻¹ s⁻¹) | kint,pred (M⁻¹ s⁻¹) |
| --- | --- | --- | --- | --- |
| 2‑methyl imidazole | 54 ± 8 | 0.18 ± 0.02 | 33 | 647 |
| Imidazole | 44 ± 9 | 0.16 ± 0.02 | 27 | 537 |
| 1‑methyl imidazole | 45 ± 8 | 0.18 ± 0.01 | 26 | 511 |
| 2,6‑dichloropurine | 29 ± 8 | 0.12 ± 0.03 | 16 | 313 |

However, in all four cases, the experimental values of kobs are approximately one order of magnitude lower than the predicted values. Each experiment was conducted at a 4:1 ligand-to-zinc ratio. These conditions were originally assumed to yield ZnL₃(OH⁻) as the dominant active species and were modeled as such. The discrepancy raised an important mechanistic question: are the lower-than-predicted rates due to breakdown of the model or to solution speciation deviating from the ZnL₃(OH⁻) species with labile monodentate Zn-ligand systems?

We probed the difference in rates in Table 2 by measuring the values of kint with 1‑methylimidazole as the ligand and ligand:zinc molar ratios ranging from 3:1 to 250:1 at pH 9, CO₂ saturation, and zinc concentrations of 1, 0.5, and 0.25 mM (see Experimental Procedures and Supplemental Information for further details). Figure 4A&B reveal a skewed bell‑shaped profile of kobs: the activity increased sharply with the amount of excess ligand, reached a maximum of ~ 440 M⁻¹ s⁻¹ at a 15:1 ratio of ligand to metal, then declined gradually. Modeling of the speciation of Zn complexes based on literature-derived observed species and values of stepwise log K’s and pKa’s for 1‑methylimidazole (1‑MIm)  reported by Appleton et al.40 indicate that a total of ~32% of zinc in solution exists as an active catalyst of some form of ligand coordination out of observed speciations at our observed catalytically optimum ratio (15:1). These experimental equilibria include species up to ZnL₆²⁺, of which ZnL₂(OH), ZnL₃(OH), and ZnL₄(OH) are the predominant hydroxo forms at pH 9, in which Lx represents the number of complexed monodentate ligands. We compute that 10% of the zinc complexes are ZnL2(OH) with water completing the tetrahedral geometry, ~3% is in the form of ZnL3(OH), and ~19% exists as ZnL₄(OH) (see SI for mass balances and algorithms, see Appleton et al.40 for reported experimental species observations). This distribution of species equates to a predicted rate constant of ~300 M⁻¹ s⁻¹ determined by equation 2:

kpred, solution = Σ fi · ki,pred	(2)

where kpred,solution is the speciation-weighted predicted rate constant for solution; fi is the equilibrium molar fraction of total active Zn present in microstate i (e.g., ZnL2(OH), ZnL3(OH), ZnL4(OH)); and ki,pred is the rate constant predicted for each microstate i.

Each species ZnL2(OH), ZnL3(OH), and ZnL₄(OH) is predicted to react with distinct rate constants of 30, 540, and 1490 M⁻¹ s⁻¹ respectively (from ΔGbind). When corrected for amount of active ligand present at 15:1 solution, ~3, ~15, and ~280 M⁻¹ s⁻¹ are predicted respectively (from equation 2). Figure 4 shows these predicted results in context with the experimental values produced for all studied compositions. The active form intrinsic rate constant predicted for ZnL₄(OH) exceeds the highest reported value (832 M⁻¹ s⁻¹) for the tri- and tetradentate ligand systems that dominate the literature. This suggests that simple monodentate scaffolds possess untapped catalytic potential currently masked by unfavorable speciation. The intrinsic rate predicted for ZnL₄(OH) (~1490 M⁻¹s⁻¹) exceeds the ~10³ M⁻¹s⁻¹ ceiling from Equation 1, which was derived from conformationally constrained multidentate systems. The flexibility of monodentate assemblies likely permits reorganization during catalysis, enabling access to more favorable ΣE(2) (enhanced donor–acceptor stabilization) and lower %Vburied values than the ground-state geometries of rigid scaffolds allow.

Figure 4. Experimentally observed rate constants for 1‑MIm plotted as a function of ligand‑to‑zinc molar ratio under constant conditions (pH 9, CO₂‑saturated, 25 °C) (solid circles). Predicted rate constants from (A) speciation model, WFT-based intrinsic rate constant predictions, and Henderson-Hasselbalch corrections (solid line); (B) includes the damping correction for free ligand concentration ([L]free). [L]free is represented by the dashed line in (B) as determined from a mass balance.

When the speciation is fully considered and applied to the results predicted for 1‑MIm in Table 2, the order of magnitude discrepancy at 3:1 ligand:zinc is significantly reduced. A population-weighted correction factor Φspec(4:1) ≈ 0.05 (26/511 M⁻¹ s⁻¹), derived from the ratio of the averaged speciation corrected rate (26 M⁻¹ s⁻¹) to predicted ZnL3(OH) rate constant (511 M⁻¹ s⁻¹), was applied to adjust predicted values for similar imidazolic monodentate ligands assuming similar speciation distributions. This corrected slate of values is found in Table 2 under the column “kint,pred,speciation corrected (M⁻¹ s⁻¹)”. The consideration of speciation corrections for computational microkinetic modeling is not new;15,41,42 however, to the best of the authors knowledge, for carbonic anhydrase mimics it has been constrained to a single, stable tri+ dentate macrocyclic chelating ligand and purely limited to a Henderson-Hasselbalch correction. This treatment is insufficient for the complexity of dynamic monodentate systems with appreciable ligand binding equilibria.

The remaining discrepancy between predicted and measured values at high ligand:Zn ratios likely arises from kinetic competition not captured by the equilibrium speciation model: free ligand can coordinate to transiently open sites during turnover, competing with CO₂ binding and effectively inhibiting catalysis beyond what the equilibrium species distribution predicts. This interpretation is consistent with previous studies suggestive of secondary interactions between product and functional groups13 and amino-acid residues in CA.43–45 The difference between the predictive curves in Figures 4A and B arises from an applied damping correction to the right portion of the curve, representing an empirical adjustment for the observed rate decline at high free-ligand concentrations when likely competes with CO₂ for transiently open coordination sites during turnover. Although the equilibrium constants describe the average distribution of species across all coordination states, the damping correction accounts for additional inhibitory effects that become significant under ligand-rich conditions.

FUNCTIONALIZATION OF CYCLEN LIGANDS

To assess how systematic covalent scaffold modification can influence intrinsic rate constants, a series of 18 mono-substituted Zn–cyclen complexes were examined across a range of electron withdrawing and donating functional groups. Each ligand features a single pendant group attached to a nitrogen donor atom, preserving the tetraazamacrocyclic coordination core. Rate constants were predicted using the ΔGbind → kint model represented in Figure 2 for all 17 compounds and expressed relative to baseline Zn–cyclen activity (set to 100%). The results plotted in Figure 5 span a range of substitutions and functional classes.

Figure 5. Predicted relative intrinsic rate constants for a series of N-substituted Zn–cyclen complexes. Percent values represent Δkint relative to baseline Zn–cyclen (set to 100%), calculated using ΔGbind and the linear correlation established in Figure 2. Substituents that modulate local solvation (e.g., alkyl chains) or enable hydrogen bonding (e.g., para-methoxybenzyl) yielded the strongest enhancements. 100% represents the baseline control. Images of the baseline ligand (cyclen) and the n-butyl modulated ligand (1-butylcyclen) are shown.

Substituents leading to enhanced predicted rates fell into two categories: alkyl chains (e.g., butyl, propyl) and electron-rich aryl groups (e.g., para-methoxybenzyl, para-hydroxybenzyl). The substituents with the largest influence are predicted to increased activity by 30-40% over the kint of the zinc-cyclen complex. In contrast, the N-pyridinylmethyl substituent is predicted to lead to a 44% decrease in rate. These effects were mirrored by modest changes in the Zn natural bonding orbital charge (NBO), which correlated with the rate when two outliers were excluded (R² = 0.73), indicating that substituent effects operate primarily through electronic modulation of the metal center.

To test these electronic predictions, rate constants were measured for four N-substituted cyclen complexes: Zn–cyclen (unsubstituted), N-butyl-cyclen, N-hexyl-cyclen, and N-benzyl-cyclen. Within the N-functionalized cyclen series, ΔGbind predicts a range of activities (±30–40% relative to cyclen, Figure 5) that do not follow the experimental ranking (predicted: Hexyl > Butyl > Benzyl > Cyclen; observed: Cyclen > Hexyl > Benzyl > Butyl). Ground-state %Vburied trends inversely with activity as seen in Table 3. The baseline cyclen (53.6%) is experimentally most active and the sterically encumbered N-butyl derivative (63.6%) is experimentally least active. This outcome contrasts sharply with the monodentate imidazole series (Table 2), in which ΔGbind preserves rank order despite comparable electronic perturbations.

Table 3: Experimental intrinsic rate constants (kobs), electronic predictions from ΔGbind, and ground-state steric congestion (%Vburied) for N-functionalised cyclen complexes (pH 9, 25 °C).

| Ligand | kobs (M⁻¹ s⁻¹) | kint, pred (M⁻¹ s⁻¹) | %Vburied |
| --- | --- | --- | --- |
| Cyclen | 343 ± 47 | 313 | 53.6 |
| Hexyl | 288 ± 63 | 393 | 62.2 |
| Benzyl | 249 ± 36 | 321 | 63.0 |
| Butyl | 182 ± 32 | 380 | 63.6 |

We attribute this divergence to cyclen scaffold rigidity. In flexible monodentate systems the ligand environment can more readily reorganize during catalysis. Here ground-state geometry does not determine the steric profile of the transition state. In contrast, the tetradentate cyclen framework is conformationally constrained, and pendant N-substituents create persistent occlusion that cannot be relieved readily by conformational changes. Under these conditions, ground-state %Vburied becomes more kinetically relevant, overriding electronic predictions. This conditional limitation does not diminish the utility of ΔGbind for screening across diverse scaffolds or within flexible ligand families but warrants explicit steric evaluation when optimizing rigid macrocyclic systems.

Taken together, these results provide a framework for tuning the rate constants in CA and other enzyme-mimetic systems via first-sphere, speciation, and functional tuning effects. The computational protocols and validations enable future database generation for high throughput screenings and machine learning, thus upgrading the subfield of biomimetic catalysts. In a broader scope, the integration of quantum chemical descriptors with quantitative speciation modeling demonstrates a transferable approach for studying ligand-dependent reactivity in multicomponent solution-phase systems.

# CONCLUSIONS

The descriptor‐driven procedure presented here links DLPNO-refined Zn-bicarbonate binding energies to intrinsic catalytic rates with precision, reproducing a five-ligand training set (R² = 0.91) and an eleven-ligand external validation set (R² = 0.90) under standardized conditions. This correlation confirms bicarbonate release (as opposed to CO₂ hydration) as the turnover-limiting step across diverse coordination environments and establishes a framework for future screening procedures.

First-sphere descriptors delineate an activity ceiling near 10³ M⁻¹ s⁻¹; within that envelope, structure still matters. Computational substitution on cyclen suggests ±30–40 % tunability by modest changes to sterics and electronics, providing an immediately actionable axis for scaffold optimization that is already captured by ΔGbind-based screening. Within conformationally flexible systems like monodentate imidazoles, ΔGbind remains predictive. However, our results indicate that in the rigid N-functionalized cyclen series, ground-state steric congestion (%Vburied) overrides electronic effects, a divergence consistent with the inability of macrocyclic scaffolds to easily reorganize during turnover. This conditional limitation defines a boundary for purely electronic screening and motivates explicit steric evaluation when optimizing within constrained ligand families.

Explicit speciation modeling reveals fundamental efficiency limitations in monodentate ligand systems: at the optimum 15:1 ratio of ligand to Zn, only ~32 % of zinc contributes to active catalysis, and only 19% is in the most active form. Together, the validated ΔGbind descriptor, quantitative speciation corrections, and demonstration of scaffold tunability (±30–40% via cyclen substitution) establish a coherent roadmap for pushing inorganic coordination complex systems beyond their traditionally observed limits through use of computational modeling and predictions. This framework aligns theory and experiment across denticities and scaffolds, prioritizes reproducibility through standardized conditions, and supplies a sturdy foundation for high-throughput screening and data-driven design of zinc catalysts for CO₂ hydration.

ACKNOWLEDGEMENTS

We are grateful to Fifth Generation, Inc. and to the Department of Energy, Office of Science, Office of Basic Energy Sciences, Chemical Sciences, Geosciences, and Biosciences Division under Contract DE-AC02-05CH11231, FWP CH030201 (Catalysis Research Program) for financial support of this work. Stopped-flow kinetics were measured in the LBNL Catalysis Laboratory.

Data-Availability Statement

All data underlying this study are available at the Dryad repository, DOI: 10.12345/xyz123.

Conflict-of-Interest Statement

The authors declare no competing financial or personal interests.

Author Contributions: D.S.C supervised this work. T.Q. prepared the manuscript. D.S.C., T.Q., C.C., and J.F.H. edited the manuscript. T.Q. developed the methodology, performed the modeling, and analyzed the kinetic data. C.C. prepared the Zn compounds and carried out the kinetic measurements.

REFERENCES

(1)	Petrik, I. D.; Liu, J.; Lu, Y. Metalloenzyme Design and Engineering through Strategic Modifications of Native Protein Scaffolds. Curr. Opin. Chem. Biol. 2014, 0, 67–75. https://doi.org/10.1016/j.cbpa.2014.01.006.

(2)	Valdez, C. E.; Smith, Q. A.; Nechay, M. R.; Alexandrova, A. N. Mysteries of Metals in Metalloenzymes. Acc. Chem. Res. 2014, 47 (10), 3110–3117. https://doi.org/10.1021/ar500227u.

(3)	Himiyama, T.; Okamoto, Y. Artificial Metalloenzymes: From Selective Chemical Transformations to Biochemical Applications. Molecules 2020, 25 (13), 2989. https://doi.org/10.3390/molecules25132989.

(4)	Zhou, Y.; Wang, Y.; Chen, Z.; Gong, H.; Chen, L.; Yu, H. Highly Hydrophilic ZIF-8 as a Carbonic Anhydrase Mimetic Catalyst for Promoting CO2 Absorption. J. Phys. Chem. C 2023, 127 (15), 7184–7196. https://doi.org/10.1021/acs.jpcc.3c00345.

(5)	Angeli, A.; Carta, F.; Supuran, C. T. Carbonic Anhydrases: Versatile and Useful Biocatalysts in Chemistry and Biochemistry. Catalysts 2020, 10 (9), 1008. https://doi.org/10.3390/catal10091008.

(6)	Lindskog, S. Structure and Mechanism of Carbonic Anhydrase. Pharmacol. Ther. 1997, 74 (1), 1–20. https://doi.org/10.1016/S0163-7258(96)00198-2.

(7)	Domsic, J. F.; Avvaru, B. S.; Kim, C. U.; Gruner, S. M.; Agbandje-McKenna, M.; Silverman, D. N.; McKenna, R. Entrapment of Carbon Dioxide in the Active Site of Carbonic Anhydrase  II. J. Biol. Chem. 2008, 283 (45), 30766–30771. https://doi.org/10.1074/jbc.M805353200.

(8)	Floyd, W. C. I.; Baker, S. E.; Valdez, C. A.; Stolaroff, J. K.; Bearinger, J. P.; Satcher, J. H. Jr.; Aines, R. D. Evaluation of a Carbonic Anhydrase Mimic for Industrial Carbon Capture. Environ. Sci. Technol. 2013, 47 (17), 10049–10055. https://doi.org/10.1021/es401336f.

(9)	Verma, M.; Bhaduri, G. A.; Phani Kumar, V. S.; Deshpande, P. A. Biomimetic Catalysis of CO2 Hydration: A Materials Perspective. Ind. Eng. Chem. Res. 2021, 60 (13), 4777–4793. https://doi.org/10.1021/acs.iecr.0c06203.

(10)	Brown, R. S.; Salmon, D.; Curtis, N. J.; Kusuma, S. Carbonic Anhydrase Models. 4. [Tris[(4,5-Dimethyl-2-Imidazolyl)Methyl]Phosphine Oxide]Cobalt(2+), a Small-Molecule Mimic of the Spectroscopic Properties of Cobalt(II) Carbonic Anhydrase. J. Am. Chem. Soc. 1982, 104 (11), 3188–3194. https://doi.org/10.1021/ja00375a043.

(11)	Kimura, E.; Shiota, T.; Koike, T.; Shiro, M.; Kodama, M. A Zinc(II) Complex of 1,5,9-Triazacyclododecane ([12]aneN3) as a Model for Carbonic Anhydrase. J. Am. Chem. Soc. 1990, 112 (15), 5805–5811. https://doi.org/10.1021/ja00171a020.

(12)	Ma, J.; Sun, N.; Zhang, X.; Zhao, N.; Xiao, F.; Wei, W.; Sun, Y. A Short Review of Catalysis for CO2 Conversion. Catal. Today 2009, 148 (3), 221–231. https://doi.org/10.1016/j.cattod.2009.08.015.

(13)	Park, D.; Lee, M. S. Kinetic Study of CO2 Hydration by Small-Molecule Catalysts with A Second Coordination Sphere That Mimic the Effect of the Thr-199 Residue of Carbonic Anhydrase. Biomimetics 2019, 4 (4), 66. https://doi.org/10.3390/biomimetics4040066.

(14)	Rains, J. G. D.; O’Donnelly, K.; Oliver, T.; Woscholski, R.; Long, N. J.; Barter, L. M. C. Bicarbonate Inhibition of Carbonic Anhydrase Mimics Hinders Catalytic Efficiency: Elucidating the Mechanism and Gaining Insight toward Improving Speed and Efficiency. ACS Catal. 2019, 9 (2), 1353–1365. https://doi.org/10.1021/acscatal.8b04077.

(15)	Koziol, L.; Valdez, C. A.; Baker, S. E.; Lau, E. Y.; Floyd, W. C. I.; Wong, S. E.; Satcher, J. H. Jr.; Lightstone, F. C.; Aines, R. D. Toward a Small Molecule, Biomimetic Carbonic Anhydrase Model: Theoretical and Experimental Investigations of a Panel of Zinc(II) Aza-Macrocyclic Catalysts. Inorg. Chem. 2012, 51 (12), 6803–6812. https://doi.org/10.1021/ic300526b.

(16)	Jiang, J.; Zhu, Q.; Huang, Y.; Zhou, D.; Zhao, L.; Guo, L.; Yang, R.; Sun, Z.; Luo, M.; Zhang, F.; Xiao, H.; Tang, X.; Zhang, X.; Song, T.; Li, X.; Chong, B.; Zhou, J.; Zhang, Y.; Zhang, B.; Cao, J.; Zhang, G.; Wang, S.; Ye, G.; Zhang, W.; Zhao, H.; Cong, S.; Li, H.; Shang, W.; Luo, Y. Automated and Intelligent Synthesis of Oxygen-Producing Catalysts from Martian Meteorites by Robotic AI-Chemist. Research Square March 23, 2023. https://doi.org/10.21203/rs.3.rs-2681831/v1.

(17)	Park, Y.; Hwang, C.-K.; Bang, K.; Hong, D.; Nam, H.; Kwon, S.; Yeo, B. C.; Go, D.; An, J.; Ju, B.-K.; Kim, S. H.; Byun, J. Y.; Lee, S. Y.; Kim, J. M.; Kim, D.; Han, S. S.; Lee, H. M. Machine Learning Filters out Efficient Electrocatalysts in the Massive Ternary Alloy Space for Fuel Cells. Appl. Catal. B Environ. 2023, 339, 123128. https://doi.org/10.1016/j.apcatb.2023.123128.

(18)	Chen, L.; Chen, Z.; Yao, X.; Su, B.; Chen, W.; Pang, X.; Kim, K.-S.; Singh, C. V.; Zou, Y. High-Entropy Alloy Catalysts: High-Throughput and Machine Learning-Driven Design. J. Mater. Inform. 2022, 2 (4), N/A-N/A. https://doi.org/10.20517/jmi.2022.23.

(19)	Liu, M.; Nazemi, A.; Taylor, M. G.; Nandy, A.; Duan, C.; Steeves, A. H.; Kulik, H. J. Large-Scale Screening Reveals That Geometric Structure Matters More Than Electronic Structure in the Bioinspired Catalyst Design of Formate Dehydrogenase Mimics. ACS Catal. 2022, 12 (1), 383–396. https://doi.org/10.1021/acscatal.1c04624.

(20)	Lan, G.; Fan, Y.; Shi, W.; You, E.; Veroneau, S. S.; Lin, W. Biomimetic Active Sites on Monolayered Metal–Organic Frameworks for Artificial Photosynthesis. Nat. Catal. 2022, 5 (11), 1006–1018. https://doi.org/10.1038/s41929-022-00865-5.

(21)	Kim, J. Y.; Kulik, H. J. When Is Ligand pKa a Good Descriptor for Catalyst Energetics? In Search of Optimal CO2 Hydration Catalysts. J. Phys. Chem. A 2018, 122 (18), 4579–4590. https://doi.org/10.1021/acs.jpca.8b03301.

(22)	Ma, R.; Schuette, G. F.; Broadbelt, L. J. Microkinetic Modeling of CO2 Hydrolysis over Zn-(1,4,7,10-Tetraazacyclododecane) Catalyst Based on First Principles: Revelation of Rate-Determining Step. J. Catal. 2014, 317, 176–184. https://doi.org/10.1016/j.jcat.2014.06.001.

(23)	Reed, A. E.; Weinstock, R. B.; Weinhold, F. Natural Population Analysis. J. Chem. Phys. 1985, 83 (2), 735–746. https://doi.org/10.1063/1.449486.

(24)	Glendening, E. D.; Landis, C. R.; Weinhold, F. Natural Bond Orbital Methods. WIREs Comput. Mol. Sci. 2012, 2 (1), 1–42. https://doi.org/10.1002/wcms.51.

(25)	Neese, F. The ORCA Program System. WIREs Comput. Mol. Sci. 2012, 2 (1), 73–78. https://doi.org/10.1002/wcms.81.

(26)	Neese, F.; Wennmohs, F.; Becker, U.; Riplinger, C. The ORCA Quantum Chemistry Program Package. J. Chem. Phys. 2020, 152 (22), 224108. https://doi.org/10.1063/5.0004608.

(27)	Aksu, G. O.; Daglar, H.; Altintas, C.; Keskin, S. Computational Selection of High-Performing Covalent Organic Frameworks for Adsorption and Membrane-Based CO2/H2 Separation. J. Phys. Chem. C 2020, 124 (41), 22577–22590. https://doi.org/10.1021/acs.jpcc.0c07062.

(28)	Clavier, H.; Nolan, S. P. Percent Buried Volume for Phosphine and N-Heterocyclic Carbene Ligands: Steric Properties in Organometallic Chemistry. Chem. Commun. 2010, 46 (6), 841–861. https://doi.org/10.1039/B922984A.

(29)	Neese, F.; Wennmohs, F.; Becker, U.; Riplinger, C. The ORCA Quantum Chemistry Program Package. J. Chem. Phys(26)	Neese, F.; Wennmohs, F.; Becker, U.; Riplinger, C. The ORCA Quantum Chemistry Program Package. J. Chem. Phys., 2020, 152, Art. No. L224108. https://doi.org/10.1063/5.0004608.

(30)	Sobez, J.-G.; Reiher, M. Molassembler: Molecular Graph Construction, Modification, and Conformer Generation for Inorganic and Organic Molecules. J. Chem. Inf. Model. 2020, 60 (8), 3884–3900. https://doi.org/10.1021/acs.jcim.0c00503.

(31)	Hanwell, M. D.; Curtis, D. E.; Lonie, D. C.; Vandermeersch, T.; Zurek, E.; Hutchison, G. R. Avogadro: An Advanced Semantic Chemical Editor, Visualization, and Analysis Platform. J. Cheminformatics 2012, 4 (1), 17. https://doi.org/10.1186/1758-2946-4-17.

(32)	GFN2-xTB—An Accurate and Broadly Parametrized Self-Consistent Tight-Binding Quantum Chemical Method with Multipole Electrostatics and Density-Dependent Dispersion Contributions | Journal of Chemical Theory and Computation. https://pubs.acs.org/doi/10.1021/acs.jctc.8b01176 (accessed 2025-05-09).

(33)	Pracht, P.; Grimme, S.; Bannwarth, C.; Bohle, F.; Ehlert, S.; Feldmann, G.; Gorges, J.; Müller, M.; Neudecker, T.; Plett, C.; Spicher, S.; Steinbach, P.; Wesołowski, P. A.; Zeller, F. CREST—A Program for the Exploration of Low-Energy Molecular Chemical Space. J. Chem. Phys. 2024, 160 (11), 114110. https://doi.org/10.1063/5.0197592.

(34)	Cousins, K. R. ChemDraw Ultra 9.0.  CambridgeSoft, 100 CambridgePark Drive, Cambridge, MA 02140. Www. Cambridgesoft.Com. See Web Site for Pricing Options. J. Am. Chem. Soc. 2005, 127 (11), 4115–4116. https://doi.org/10.1021/ja0410237.

(35)	Poater, A.; Cosenza, B.; Correa, A.; Giudice, S.; Ragone, F.; Scarano, V.; Cavallo, L. SambVca: A Web Application for the Calculation of the Buried Volume of N-Heterocyclic Carbene Ligands. Eur. J. Inorg. Chem. 2009, 2009 (13), 1759–1766. https://doi.org/10.1002/ejic.200801160.

(36)	Wistrand, P. J. The Importance of Carbonic Anhydrase B and C for the Unloading of CO2 by the Human Erythrocyte. Acta Physiol. Scand. 1981, 113 (4), 417–426. https://doi.org/10.1111/j.1748-1716.1981.tb06918.x.

(37)	Yadav, N.; Singh Chundawat, T. Recent Trends on Zinc Complexes Bearing Bi-, Tri- and Tetra-Dentate Schiff Base Ligands for Catalytic Ring-Opening Polymerization of Lactides. ChemistrySelect 2024, 9 (34), e202401619. https://doi.org/10.1002/slct.202401619.

(38)	Huang, D.; Makhlynets, O. V.; Tan, L. L.; Lee, S. C.; Rybak-Akimova, E. V.; Holm, R. H. Fast Carbon Dioxide Fixation by 2,6-Pyridinedicarboxamidato-Nickel(II)-Hydroxide Complexes: Influence of Changes in Reactive Site Environment on Reaction Rates. Inorg. Chem. 2011, 50 (20), 10070–10081. https://doi.org/10.1021/ic200942u.

(39)	Zdrazil, B.; Felix, E.; Hunter, F.; Manners, E. J.; Blackshaw, J.; Corbett, S.; de Veij, M.; Ioannidis, H.; Lopez, D. M.; Mosquera, J. F.; Magarinos, M. P.; Bosc, N.; Arcila, R.; Kizilören, T.; Gaulton, A.; Bento, A. P.; Adasme, M. F.; Monecke, P.; Landrum, G. A.; Leach, A. R. The ChEMBL Database in 2023: A Drug Discovery Platform Spanning Multiple Bioactivity Data Types and Time Periods. Nucleic Acids Res. 2024, 52 (D1), D1180–D1192. https://doi.org/10.1093/nar/gkad1004.

(40)	Appleton, D. W.; Sarkar, B. The Activity-Related Ionization in Carbonic Anhydrase. Proc. Natl. Acad. Sci. U. S. A. 1974, 71 (5), 1686–1690. https://doi.org/10.1073/pnas.71.5.1686.

(41)	Ma, R.; Schuette, G. F.; Broadbelt, L. J. Insights into the Relationship of Catalytic Activity and Structure: A Comparison Study of Three Carbonic Anhydrase Mimics. Int. J. Chem. Kinet. 2014, 46 (11), 683–700. https://doi.org/10.1002/kin.20879.

(42)	Ma, R.; Schuette, G. F.; Broadbelt, L. J. Catalyst Screening through Quantum Chemical Calculations and Microkinetic Modeling: Hydrolysis of Carbon Dioxide. Theor. Found. Chem. Eng. 2017, 51 (6), 949–960. https://doi.org/10.1134/S0040579517060124.

(43)	Paul, T. K.; Taraphder, S. Coordination Dynamics of Zinc Triggers the Rate Determining Proton Transfer in Human Carbonic Anhydrase II. ChemPhysChem 2020, 21 (13), 1455–1473. https://doi.org/10.1002/cphc.202000177.

(44)	Zhang, L.; Xie, D. Density Functional Theory Study of Proton Transfer in Carbonic Anhydrase. Chin. Sci. Bull. 2005, 50 (22), 2557–2559. https://doi.org/10.1360/982005-485.

(45)	Fisher, Z.; Hernandez Prada, J. A.; Tu, C.; Duda, D.; Yoshioka, C.; An, H.; Govindasamy, L.; Silverman, D. N.; McKenna, R. Structural and Kinetic Characterization of Active-Site Histidine as a Proton Shuttle in Catalysis by Human Carbonic Anhydrase II,. Biochemistry 2005, 44 (4), 1097–1105. https://doi.org/10.1021/bi0480279.